Godsgift Arokarawei
 Southern New Hampshire University
 CS 350
 Instructor: Toni Farley
 August 11, 2025

Reflection on the Thermostat Final Project

The thermostat final project was a valuable opportunity to bring together concepts from embedded systems, Python programming, and hardware design into a single cyber-physical system. My solution followed the project lab guide but also incorporated enhancements for clarity, reliability, and functionality. The system consisted of a Raspberry Pi connected to an AHT20 temperature and humidity sensor via the I²C bus, a 16×2 LCD for real-time data display, two LEDs to indicate heating and cooling modes, and three push buttons for user input—an approach similar to other programmable thermostat designs (Vasiliu et al., 2015).

The program logic was implemented using a finite state machine (FSM) with three primary states: OFF, HEATING, and COOLING. The system compared the current temperature reading from the AHT20 sensor to the user-defined setpoint. When the temperature was below the setpoint in heating mode, the heating LED illuminated; when above in cooling mode, the cooling LED activated. The push buttons allowed mode selection and setpoint adjustment in single-degree increments. The FSM ensured predictable state transitions and prevented conflicting actions, aligning with established thermostat control strategies (Allen et al., 2015).

One of the biggest challenges was button debounce. Initially, a single press would register multiple times due to mechanical switch bounce. I addressed this in software by adding a short delay and validating the button state after the delay. Another challenge was ensuring stable I²C communication. Loose wiring or incorrect sensor addresses could cause runtime errors. I used i2cdetect to verify device addresses and wrapped sensor reads in try/except blocks to handle communication errors gracefully, a method also recommended in IoT device reliability research (Atabekov et al., 2015).

To meet the UART telemetry requirement, the system logs each cycle’s timestamp, current temperature, humidity, setpoint, and active mode to the console. This output could be redirected to a .txt file, creating a data record for performance monitoring or troubleshooting.

Through this project, I gained a deeper understanding of how hardware and software interact in real time. The FSM approach made the thermostat behavior clear and predictable, while working with actual components reinforced my knowledge of electronic interfacing, bus communication protocols, and physical device constraints. I also improved my Python coding practices, focusing on modular functions, clear variable naming, and maintainable structure.

This project closely mirrors real-world thermostat design, where reliability, safety, and responsiveness are critical. The skills learned here—debugging hardware, designing state logic, and integrating data logging—are directly applicable to professional embedded systems engineering.

References

Allen, M., Billett, S., & Read, K. (2015). Smart thermostat. https://scholarcommons.scu.edu/elec_senior/16/

Atabekov, A., Starosielsky, M., Lo, D. C. T., & He, J. S. (2015, July). Internet of Things-based temperature tracking system. In 2015 IEEE 39th Annual Computer Software and Applications Conference (Vol. 3, pp. 493–498). IEEE. https://doi.org/10.1109/COMPSAC.2015.261

Vasiliu, R. N., Popa, M., & Marcu, M. (2015, November). Wireless Programmable Thermostat Using Raspberry Pi. In Soft Computing Applications: Proceedings of the 6th International Workshop Soft Computing Applications (SOFA 2014), Volume 2 (pp. 949–957). Cham: Springer International Publishing. https://link.springer.com/chapter/10.1007/978-3-319-18416-6_75